Contamination Screening Station Job Action Sheet

This station is in the Contamination Control Zone. Workers must wear PPE assigned to them.

Before Shift

- Report to the Staff Sign-In Area

- Collect assigned personal protective equipment (PPE), dosimetry devices, and radiation detection instruments

- Attend pre-shift briefing

- Report to the Contamination Screening Station Manager to receive and review your station assignment

  - After receiving your station assignment, review the appropriate Job Aid

| Station Assignment | Screening Assignment Job Aid |
| --- | --- |
| Initial Sorting | High Contamination Screening |
| Contamination Screening | G-M Detector Job Aid Portal Monitor Job Aid |
| Wash Station | G-M Detector Job Aid Portal Monitor Job Aid |

During Shift

- Report any emergencies or injuries to your station manager IMMEDIATELY

- Wear assigned PPE and dosimetry devices at all times

- As possible, avoid physical contact with arrivals to limit cross-contamination

  - If you touch someone, change your gloves or have them checked for contamination

- Screen people for external contamination

  - Follow screening protocol established by Contamination Screening Station Manager

- As necessary, escort people to the appropriate station

During Shift (Continued)

- Direct people as follows:

| Decision |  | Action |
| --- | --- | --- |
| Contaminated |  | Escort to Wash Station |
| Not Contaminated |  | Direct to Registration |

- Conduct contamination screenings for workers (including other Contamination Screening Staff)

    - Direct them on how to remove their PPE

    - Screen them for contamination

After Shift

- Report to the Contamination Screening Station to begin demobilization

  - Contamination Screening Staff will:

    - Direct you on how to remove your PPE

    - Screen you for contamination

- After entering the Clean Zone:

    - Turn in your dosimetry devices and radiation detection instruments at the Sign-Out Area.

    - Attend the post-shift briefing